| # Piper Warrior II Checklist |  |  |  | ##### START-UP (cont.) Mixture – Lean Avionics – On Trnspnder: STBY, squawk Check ATIS / AWOS Altimeter – Set Call GND (Com2) # TAXI Lights – as Req. Brakes – Test In Turns Check: Compass, AI, DG, TC # BEFORE RUN-UP Brakes – Set Fuel – Select Fullest Tank Flight Controls – Check Instruments – Check Autopilot – Test Annunciators – Check Primer – In & Locked | # RUN-UP Mixture – Best Power 2000 RPM Mags (R & L) – Test Carb Heat – Test Vacuum Amps / Volts Fuel Pump – On Fuel Pressure – Check Fuel Pump - Off Oil Pressure/Temperature 1000 RPM Mixture – Lean # BEFORE TAKEOFF Autopilot – Off Air Conditioner – Off Flaps – As required Trim – Takeoff Carb Heat – Off Fuel Pump – On Set DG to Compass Doors / Windows Pitot Heat – As required Brakes – Release Radio – Call TWR (Com1) 3 |
| --- | --- | --- | --- | --- | --- |
| VR rotate 55 VX best angle 63 VY best rate 79 VG best glide 73 - 61 VA maneuvering (1531 lbs) 93 (Full Gross) 111 | VS0 stall dirty 44 VS stall clean 50 VNO structural cruising 126 VNE never exceed 160 VFE flap extension 103 Max demo’d xwind 17 1.2 * VS0 53 1.3 * VS0 57 |  |  |  |  |
| Short Field Takeoff 25 Flaps (two notches) Rotate 50 Then 63 Gradually retract flaps | Soft Field Takeoff 25 Flaps (two notches) Stay in ground effect to 63 Climb at VX or VY Gradually retract flaps |  |  |  |  |
| CRUISE (TAS @ 5,000’) Economy 93 2310 RPM – 7.8 GPH – 55% Normal 103 2450 RPM – 8.8 GPH – 65% Maximum 113 2595 RPM – 10.0 GPH – 75% |  |  |  |  |  |
| ARRIVAL Speed: Flaps: Normal Approach 70kts 25o – 40o Short Field Approach 63kts 40o 1 |  |  |  |  |  |
| # INTERIOR Weather, Density Altitude Weight & Balance AROW Check headset battery Locate Fire Extinguisher Locate Emergency Checklist Flaps – Extend Master – On Fuel Gauges – Check Pitot Heat – Test #### Stall Warning – Test Lights – Check Master – Off # EXTERIOR Fuel Quantity & Quality Caps / Drains / Vents Engine / Oil / Belt Prop / Air Intake Exhaust System Surfaces and Controls Pitot & Static Ports Gear / Tires / Brakes Antennas Ties / Chocks – Remove Baggage Door Final Walk Around |  | # BEFORE START-UP Seat and Harness – Check Flaps – Retract Check Hobbs Time Fuel – Least Full Tank Circuit Breakers Alternate Static ELT – Armed Brakes – Pedal Test / Set Passenger Brief # START-UP Avionics – Off Autopilot – Off Air Conditioning / Fan – Off ##### Carb Heat – Off Check Throttle Friction ##### Prime (5x for cold engine) Mixture – Full Rich Throttle – 1/8” Prop Area – Check clear Master – On Beacon – On Fuel Pump – On Brakes – Hold Start – Yell: ”Clear” 1000 RPM Oil/Fuel Pressure – Check Fuel Pump – Off 2 |  | # TAKEOFF Lights: As required Camera: Transponder – Alt Action: Mixture – Rich Throttle – Full Check: 2350 RPM (Min) Oil Pressure/Temp Fuel Pressure Rotate @ VR = 55 kts Climb @ VY = 79 kts # 1000’ Check Fuel Pump – Off Landing Light – Off Flaps – Check Up Cruise Climb @ 79 - 87 kts Power & Mixture – Set Engine instruments – Check Flight Plan – Open # CRUISE Power / Mixture Instruments – Scan Fuel – Proper Tank # DESCENT Mixture – Richen Fuel – Proper Tank Carb Heat – As required | # BEFORE LANDING Autopilot – Off Gas – Fullest Tank Fuel Pump On Mixture – Best Power Switches – Landing Light On Seat Belts / Harnesses Carb Heat – As required Short final  63 kts # AFTER LANDING Trim – Takeoff Flaps – Retract Mixture – Lean Carb Heat – Off Fuel Pump – Off - Lights – As required Pitot Heat – Off Transponder – STBY # Radio – Call GND # SECURING Switches: Avionics,AP,AC – Off Lean: Mixture (ICO) Ignition: Mags – Off Master: Off Control Lock / Pitot Cover 4 |

| # ENGINE FAILURE IMMEDIATELY AFTER TAKEOFF Maintain Control Best Glide – 73 KIAS (Full Gross Wt.) Fuel Selector - Off ##### Mixture – Full Lean / Idle Cutoff Flaps - Down #### Master & Mags – Off (Unlatch Door) ENGINE FAILURE IN FLIGHT Best Glide – 73 KIAS (Full Gross Wt.) Carb Heat – On (Also Supplies Alternate Air) Note Wind Direction & Velocity Pick Landing Site & Initiate Approach Mixture – Full Rich Fuel Pump – On Fuel Selector – Check / Switch (Note Gauges) Fuel Primer – Locked (Try Re-priming) Magnetos – Check All Master – On |  | IF NO RESTART & TIME PERMITS Maintain Best Glide Squawk 7700 Declare Emergency (TWR, APP, ATC, UNICOM, 121.5) Fuel Selector – Off ELT – On Avionics – Off Mixture – Full Lean / Idle Cutoff Seatbelts / Harness – Tight Flaps – As Needed (Full Flaps When Field Assured) Master & Mags – Off Reduce Air Speed Unlatch Door Protect Body ELECTRICAL FIRE IN FLIGHT Master – Off (Leave Mags On) All Electrical Devices – Off Vents, Cabin Heat, & Air – Off If Fire Out – Try Master On Only Then One Essential Electrical Device At A Time |  |
| --- | --- | --- | --- |
| ENGINE FIRE IN FLIGHT Throttle – Closed Mixture – Full Lean / Idle Cutoff Fuel Selector – Off Master Switch – Off Cabin Heat & Air – Off (Except Overhead Vents) Increase Airspeed as Needed to Extinguish ENGINE FIRE DURING START Continue Cranking Engine If Start – Run A Few Seconds – Shutdown – Inspect If No Start – Mixture Idle Cutoff – Fuel Selector Off Throttle Full Open Continue Cranking Engine A Few Seconds Master & Mags – Off Evacuate / Fire Extinguisher |  | ICE Pitot Heat – On Carb Heat – On (or as Req.) Cabin Heat & Defrost – MAX STRONGLY Consider 180 Turn Attain Higher or Lower Altitude Increase Engine Speed Flaps – Not Recommended for Landing Land Faster as Needed RADIO FAILURE - Check Circuit Breakers & Volume - Recycle Alternator Switch - If IFR & Radio Still Out, Set Transponder at 7600. - Suggested for VFR if in B, C, D Airspace. |  |